NeuroLearn Research Suite (NEURO_FOCUS)
Comprehensive Technical & Research Project Report
A Modular Biomedical Signal Processing (BSP) Platform for EEG-Based Attention Quantification

| Primary Dataset: PhysioNet_S001 (eegmmidb) | Methodology: 100% Non-ML / Explainable BSP |
| --- | --- |
| Sampling Rate (fs): 160.0 Hz | Channel Count: 64 Channels |
| Recording Duration: 61.0 Seconds | Quality Audit: 72.73% SQI Score |

# 1. Executive Summary & Real-World Dataset Validation

This report presents an empirical biomedical signal processing study evaluating student cognitive attention state derived directly from the PhysioNet EEG Motor Movement/Imagery Database (eegmmidb). The primary dataset consists of 64-channel EEG recordings sampled at 160.0 Hz from PhysioNet_S001 encompassing both resting baseline (Run 1) and active task conditions (Run 2).

Strict Non-ML Policy: In accordance with rigorous scientific standards, zero machine learning or black-box predictive models were used. All attention metrics were computed using zero-phase Butterworth bandpass filtering (1-40 Hz), 50 Hz powerline notch filtering, Welch Power Spectral Density (PSD), Hjorth parameters (Activity, Mobility, Complexity), and mathematical band ratio formulations (Beta/Theta).

# 2. Core Modular Architecture & Subsystems

The platform comprises 17 decoupled backend processing modules and 21 interactive frontend workspaces:

1. Dataset Manager: Parses EDF, MAT, CSV, and TXT recordings with automatic metadata detection.

2. Biopac Import Manager: Normalizes AcqKnowledge MAT/CSV exports into standard EEGData containers.

3. PhysioNet Adapter: Fetches and formats public PhysioNet EEG Motor Imagery datasets (eegmmidb).

4. Signal Quality Assessment (SQI): Computes SQI scores (0-100%), SNR (dB), and auto-excludes bad channels.

5. Signal Preprocessing Cleaner: Executes zero-phase Bandpass (1-40Hz), 50Hz Notch filter, and Z-score standardization.

6. Sliding Window Engine: Segments continuous EEG into sliding epochs (2s to 30s) with overlap.

7. Feature Extraction Engine: Extracts time-domain stats, Hjorth parameters, Welch PSD, SEF95, and Wavelet Entropy.

8. Frequency Analysis Subsystem: Calculates absolute and relative band powers for Delta, Theta, Alpha, Beta, and Gamma.

9. Mathematical Attention Calculator: Computes non-ML ratio formulas, AST user expressions, and 5-level state classification.

10. Statistical Analysis Module: Performs hypothesis testing, Shapiro-Wilk normality tests, Cohen's d, and Bland-Altman.

11. Systematic Ablation Study Engine: Evaluates Preprocessing pipelines, Window sizes, PSD methods, and Formulas.

12. Research Validation Engine: Direct cross-dataset agreement audit between PhysioNet and Biopac EEG sessions.

13. Experiment Manager & Reproducibility: Logs software versions, filter bounds, and exports 100% reproducible JSON snapshots.

14. Multi-Format Report Generator: Exports IEEE/Frontiers-styled PDF, Word DOCX, and CSV dataset reports.

# 3. Empirical Attention Analysis (Baseline vs. Task)

| Metric | Run 1 (Baseline) | Run 2 (Task) | Scientific Significance |
| --- | --- | --- | --- |
| Average Attention Score | 51.4 / 100 | 48.3 / 100 | Mean cognitive score |
| Peak Attention Score | 95.3 / 100 | 79.9 / 100 | Maximum focused state |
| Minimum Attention Score | 17.2 / 100 | 11.3 / 100 | Nadir attention level |
| Attention Stability Index | 79.8% | 78.6% | Inverse std deviation |
| Dominant Category | Low Attention | High Attention | 5-Level rule-based state |

# 4. Statistical Hypothesis Testing & Bland-Altman Agreement

Hypothesis Test: Independent Two-Sample t-Test
Test Statistic: 0.494 (p-value = 0.6240)
Cohen's d Effect Size: d = 0.15
Bland-Altman Agreement: Mean Bias = 3.09 points, 95% Limits of Agreement = [-61.87, 68.05], Pearson r = -0.218.

# 5. Real-World Systematic Ablation Study Results

| Preprocessing Pipeline | Mean Attention | Peak Attention | Stability Index | Dominant Category |
| --- | --- | --- | --- | --- |
| Raw Signal | 52.9 / 100 | 94.7 | 79.6% | High Attention |
| Bandpass Only | 52.9 / 100 | 94.7 | 79.6% | High Attention |
| Bandpass + Notch | 52.9 / 100 | 94.7 | 79.6% | High Attention |
| Full Preprocessing | 51.4 / 100 | 95.3 | 79.8% | Low Attention |

# 6. Mathematical Derivations & Academic References

• 4th Order Zero-Phase Butterworth Bandpass: |H(jw)|^2 = 1 / [ 1 + ((w^2 - w0^2)/(w*B))^(2N) ]
• Hjorth Activity: var(x(t)) = sigma_x^2 | Mobility: sigma_x' / sigma_x | Complexity: Mobility(dx/dt) / Mobility(x(t))
• Welch PSD: PSD(f) = (1/K) sum_{k=1}^K |FFT(x_k * w)|^2 / (L * U)
• Spectral Edge Frequency (SEF95): Integral_0^{f95} PSD(f) df = 0.95 * Total_Power
• Attention Ratio: Beta / Theta = Power(13-30Hz) / Power(4-8Hz)
• Cohen's d: d = (Mean_1 - Mean_2) / s_pooled

Academic References:
1. Goldberger, A. L., et al. (2000). 'PhysioBank, PhysioToolkit, and PhysioNet: Components of a new research resource for complex physiologic signals.' Circulation, 101(23), e215-e220.
2. Coelli, S., et al. (2018). 'EEG-based indices of attention during online learning tasks.' IEEE Transactions on Biomedical Engineering.
3. Hjorth, B. (1970). 'EEG analysis based on time domain properties.' Electroencephalography and Clinical Neurophysiology.